
	                                           规格书


	CUSTOMER:
	

	MODEL NUMBER:
	AC-1407A 

	OUR PART NO.:
	

	CUSTOMER PART NO.:
	

	CUSTOMER

APPROVED
CHECKED



	1. Technical Parameter
Measuring condition

Part shall be measured under a condition (Temperature: 5～35℃, Humidity: 45%～85%R.H., Atmospheric pressure: 860 ～1060hPa) unless the standard condition (Temperature: 25±3℃, Humidity: 60±10%R.H. Atmospheric pressure: 860 ～1060hPa) is regulated to measure.

	1
	Resonant Frequency
	4000±400Hz

	2
	Operating Voltage
	1～25Vp-p

	3
	Rated Current
	Max.3mA , At 4KHz 50% duty Square Wave 5Vp-p

	4
	Sound Output at 10cm
	Min. 85dB, At 4KHz 50% duty Square Wave 5Vp-p

	5
	Capacitance
	12000±30% pF At 120Hz

	6
	Operating Temperature
	-20℃～+70℃

	7
	Store Temperature
	-30℃～+80℃

	8
	Net Weight
	Approx 0.7g

	9
	RoHS
	NO

	2. Dimensions
	Unit: mm

	          
[image: image1.wmf]
	
[image: image2.emf]

	*Unit: mm; Tolerance: ±0.5mm Except Specified
*Housing Material: Black ABS


	3. Electrical And Acoustical Measuring Condition

	Recommended Setting


[image: image3.wmf]4

0

c

m

1

0

c

m

M

i

c

.

S

P

L

d

B

测

试

示

意

图

B

u

z

z

e

r

S

M

-

1

6

4

1

F

U

N

C

T

I

O

N

G

E

N

E

R

A

T

O

R

信

号

发

生

仪




	4. Frequency Response

[image: image4.png]01/02,2000 80:10 We121F g 05 B W W X | RAE HEEEL

e F= 2667 Hz 073.15 dB

ERES OB
B E:3110 mU Fo= 4019 Hz 090.40 dB

11o LB BH(i% 1 20.0 yPa_ ATER : 001 ms i SR ¢ 1% oct
® ﬁﬁﬁﬁﬁ#?

100

050

080

070

060
4k Sk 6k 7k 8k 10k

00827

200Hz 300 400 SO0 600 800 1k





          5Vp-p 50% duty Square wave,10cm


	5. Reliability Test
After any following tests the part shall meet specifications without any degradation in

appearance and performance except SPL. SPL shall not deviate more than -5 dB

from the initial value

5.1 Ordinary Temperature Life Test

The part shall be subjected to 96 hours at 25±10℃. Input rated voltage
Resonant frequency, 1/2 duty Square wave.

5.2 High Temperature Test
The part shall be capable of with standing a storage temperature of +70℃ for 96 hours.
5.3 Low Temperature Test 
The part shall be capable of with standing a storage temperature of ​-20℃ for 96 hours.
5.4 Humidity Test
Temperature:+40℃±3℃   Relative Humidity:90%～95% Duration: 48 hours

and expose to room temperature for 6 hours
5.5 Temperature Shock Test

Temperature:60℃ /2hours→ -20℃/2hours (1cycle), Total cycle: 10 cycles

5.6 Drop Test

Standard Packaging From 1.2m(Drop on hard wood or board of 5cm thick, 

three sides, six plain.)
5.7 Vibration Test

Vibration:1000cycles /min. Amplitude:1.5mm, Duration: 1 hour in each 3 axes


	Note:
As this product is not protected from foreign material entering, please make sure that any foreign materials (e.g. magnetic powder, washing solvent, flux, corrosive gas) do not enter this product in your production processes. The functional degradation (e.g. SPL down) may occur if foreign material enter it.



_1234567891

_1234567892.bin

_1234567893

_1234567890.bin

