
	[image: image4]
DATE:                          SPEC. No.: YD1511                    Total：6 sheet

SPECIFICATION 

SHEET FOR APPROVAL

(Revision：2010M  Update: 00)

客户/CUSTOMER: 
产品/PRODUCTS:  Mobile Speaker

型号/MODEL NUMBER: YD1511
客户型号/CUSTOMER NUMBER: 
产品内部描述/QUANCHENGLI INTERNAL DESCRIPTION:

15*11mm           H=3.5        L=15        
环保要求/ENVIRONMENT REQUEST：

■ ROHS             □ NON-ROHS            □ OTHERS

ENGINEER

CHECKED

APPROVED

CUSTOMER  APPROVED           

SIGNATURE
IMPRINTMENT




变更记录表/ALTERNATION HISTORY
	序号 No.
	修订本Revision
	更  新  Update
	注    释                           Comment
	变更页Page No.
	日期

Date
	设计者Design

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	
	
	
	
	
	
	

	

	Name

手机喇叭
Mobile Speaker
	客户型号
Customer’s part no:
	

	
	我司型号
Our part no:
	YD1511


	1． 范围/SCOPE
该规格书适用于移动电话机的扬声器单元。

This specification covers our product of dynamic receiver unit for mobile telephone use.
2．型号/PART No

我司型号/Customer’s Part no：

本公司型号/Our Part no  ：YD1511 L15H3.5
3．尺寸与重量/DIMENSION AND WEIGTH

尺寸：     参考外观图面 （Figure.3）
Dimension： Refer To The Appearance Drawing. （Figure. 3）
重量( Weight)：      \±0.1g
4．标准测试条件/STANDARD TEST CONDITION

在没有其他特别指定时，测试应如下所记的标准测试条件下进行。

Unless Otherwise Specified，The Test Shall Be Made Under The Standard Conditions。

·温度/Temperature         ：17℃～25℃（Standard：20.5℃）

·相对湿度/Relative Humidity ：45%~85%（Standard：65%）

·大气压力/Pressure        ：86Kpa~106Kpa(Standard：101.3 Kpa) 
If there is no doubt about the results, measurement shall be made with in the following limits.

（当判定有疑义的场合如下述测试条件）： 20℃±3℃ ，60%～70%RH ，860 HPA～1060 HPA
5． 使用与储存温度/OPERATION AND STORAGE TEMIPERATURE

    ·使用温度/Operation Temperature  ：-40℃~85℃
·储存温度/Storage Temperature    ：-30℃~ 70℃
6． 电气特性规格/ELECTRICAL SPECIFICTION (RECEIVER MODE)
参考特性规格页面/Refer To The Appearance Page
6.1 灵敏度/SOUND PRESSURE LEVEL(S.P.L) : 
88±3dB (at0.1m,0.1W)  at 0.9、1.0、1.2、1.5 kHz in Average. 
Measure Condition: sin swept measurement at 0.1w on axis at 0.1m.With IEC Baffle plate.
6.2最低共振周波数/RESONANCE FREQUENCY: 850±20% Hz  ( At 1.0 V)

6.3 额定阻抗/RATED IMPEDANCE: 8.0±15% ohm( at 1kHz 1.0V)
6.4 额定输入功率/RATED INPUT POWER : 0.5W     
6.5 最大输入功率/MAX INPUT POWER : 0.8W
6.6 异常音BUZZES & RATTLES  MUST BE NORMAL// SIN WAVE 2.0V 

6.7 总谐波失真/TOTAL HARMONIC DISTORTION: Less than 5% at 1kHz, 0.5W

6.8 有效频率范围/FREQUENCY RANGE: FO～20KHz  ( OUTPUT S.P.L±15dB)

	Name

手机喇叭
Mobile Speaker
	客户型号
Customer’s part no:
	

	
	我司型号

Our part no:
	YD1511


	RECEIVER SPECIFICATION

	■典型频响曲线 /TYPICAL FREQUENCY RESPONSE CURVE

[image: image1.emf]
Figure.1

■ 测试方框图/FREQUENCY MEASURING CIRCUIT(SPL:测试0.1M/0.1W  0.89V  IMP测试: 1V)
[image: image2.png]


 Figure.1



	Name

手机喇叭
Mobile Speaker
	客户型号
Customer’s part no:
	         

	
	我司型号
Our part no:
	YD1511


	RECEIVER  SPECIFICATION(P5/6)
7． Reliability Tests / 可靠性测试

Sound pressure level difference shall be within ±3dB after test at 1000Hz

测试时,音压值在1000Hz时在±3dB之内

7.1高温实验/High Temperature test
+80±3℃ duration 96 hours ,then removal back to normal temperature for 2 hours.       (+80±3℃，96小时实验后，常温放置2小时。)
7.2低温实验/Low Temperature Test
-40±3℃ duration 96 hours ,then removal back to normal temperature for 2 hours.      （-40±3℃， 96小时实验后，常温放置2小时。）
7.3耐湿实验/Humidity Test
Temperature +40±3℃,relative humidity 90%～95%,duration 96hours,then removal back to normal temperature for 2hours.
(温度+40±3℃，相对湿度90%～95%,96小时实验后，常温放置2小时。)
[image: image5]7.4热冲击实验/Temperature Cycle Test
Temperature : -20±3℃ to +60±3℃

Duration    : 2 hours     2 hours  

Cycle       : 2cycle
(温度-20℃2小时转

高温+60℃2小时为一周期，

共试验2周期。）
7.5 Vibration test ibration
振动试验（捆包状态）10-55Hz/min
Amplitude :  1.5mm
Duration  :  2hours each axes 

7.6跌落实验/Drop Test 
RECEIVER Shall Be Packed In The Carton And Dropped The  concrete board (H:1m) 

产品在包装状态下,从1米高处向混凝土地面跌落:1角:1次:3边:每条边1次:6面:每1面1次
7.7 寿命实验/Load Test
   Subject samples to White noise (EIA filter) for 96 hours at0.5W input power.
   产品输入0.5W的白噪声信号连续工作96小时。



	Name

手机喇叭
Mobile Speaker
	客户型号
Customer’s part no:
	        

	
	我司型号
Our part no:
	YD1511


	RECEIVER SPECIFICATION(P6/6)

	■ 产品外观图/ DIMENSIONAL DRAWING

[image: image3.jpg]11





[image: image6.wmf]+20

C

2H

2H

Hour 

C

C

1 cycle

20

～

40s

20

～

40s

+80

-40

View direction:

Figur.3

TABLE OF METERIALS 
ITEM

PARTNAME

MATERIAL

SPECIFICATION

PROVIDER

1

Receiver Unit

15*11*3H

2

Gasket

0.3T
双面背胶

3

Wire

Red and Black

Table .1

	Name

手机喇叭
Mobile Speaker
	客户型号
Customer’s part no:
	        

	
	我司型号
Our part no:
	YD1511


� EMBED  \* MERGEFORMAT ���





� EMBED  \* MERGEFORMAT ���









[image: image7.wmf]_1234567890.dwg

_1234567891.dwg

